How Dallas Key File (DKL) Works

#1, DKL File Structure

Header

Schema

Data

SHA1(PIN) (if the header doesn’t say PIN required, there is no PIN field at here)

Signature1

Signature2 (if the header says one signature, there is no signature2)

CRC

Dallas Key File Format

MXD Header

Schema

Dallas Key List 1 To N

CRC

Key Number Function

#2, The Usage of Dallas Key File

Dallas Key file stores the key IDs and usage of Key IDs, which means the special key is used for special purpose, like Vault lock, Upper lock.

#3, DKL Update Process

A Dallas Key file will be updated by service engineer manually via file copy in Supervisor Tasks. (Service Engineer select a file to copy)

#3-1, A Dallas key file is a MXD file, at the header, it requires a PIN which means the service engineer shall enter the pin when a Dallas Key file is copied to machine.

#3-2, Usually, the DKL file has a Public Key at the header, and one signature before the CRC. It also might be 2 signatures before the CRC, which one is for old PK, another is for new PK.

#3-3, The data of PIN field at MXD file is used to generate a PIN code via SHA1(PIN Data) to compare to the service engineer input.

SHA1(PIN Data)  byte[24] [0,15] is the PIN Code

A, when machine find a DKL is tried to be copied, machine shall check CRC first

B, machine reads header to know if PIN is required and 1 or 2 signatures here

C, if PIN is required, get the data of PIN field and generate PIN code via SHA (PIN) to compare to the PIN Code service engineer inputted.

D, if the PIN Code is correct, then the DKL (or DKL-MXD-Decoded) file will be copied onto machine.

E, go to verify signature with PK (which algorithm for signature?)

The machine then verifies the signature (one or two) to check to decide whether a PK shall be updated.

F, Decrypt the Schema and Data of decoded DKL file with TEAN algorithm.

#4, General Decoding Process

1, CRC removed and verified first to ensure the file is correct

2, Read header to know whether it is a PIN required as well as one or two signatures and encrypted.

3, check signature1 to ensure the file is not tampered.

4, if there is a Signature2, try to update PK

5, check PIN code if needed

6, decrypt Schema and Data with TEAN algorithm if header says it is encrypted

The decryption PIN comes from SHA1 (Header)  byte[24] [0,15]

7, read schema and data

8, update any MXD files

#5, The Decryption of DKL Schema & Data

The schema and data fields of DKL file shall be decrypted by TEAN algorithm which is implemented in class TeanDecoder of MXDPaser.C#.

There is something missed in the documentation, the decryption algorithm shall be referred to Buffer.BlockCopy of MSDParser to get correct;

The Decryption Key of TEAN algorithm is generated by SHA (Header)  byte [24],

[0,15] is the KEY of decryption.

#6, The Public Key shall be stored into Vault PCB.

At initial phase, there is no Public Key (PK) in PCB, machine checks a DKL file, and uses the PK from Header and verify the signature of DKL file. If it is verified, machine save the PK into PCB, and for later verifications.

If there are two signatures in DKL file, machine uses current PK to check the old signature first to authentic whether this file is genuine. If it is good, then the machine uses current PK to check the new one, if it is also good, it means the new PK is already stored into PCB. If the new signature is different to the calculation, machine shall use the Key of the header to verify. If the Key of Header works with new signature, machine stores the Key of Header into PCB as PK. Otherwise, discard it.

#7, How Dallas Key works

Once the IO board reads the key ID of Dallas Key from Dallas Key reader, IO board sends Key ID to Android application, Android application verifies the Key ID with DKL, and tells IO board to open the related lock if the Key ID is valid, meanwhile Android Application makes a record at app log. IO board returns the result of opening.